Hot Work Procedure

Hot Work Procedure

Purpose:

The purpose of this procedure is to outline the Occupational Health and Safety (OHS) requirements in Department of Education and Training (the Department) workplaces for hot work tasks. This procedure should be read in conjunction with the Plant and Equipment Management Procedure.

Scope:

This procedure applies to all Department workplaces, including schools and central and regional offices.

Procedure

No Department employee is permitted to undertake any hot work activities unless their role specifically requires them to do so, and have completed a risk assessment.

## Hot work task identification

The Workplace Manager and/or Management OHS Nominee, in consultation with the Health and Safety Representatives (HSR) and employees are to identify all tasks which have the potential to generate heat, flames or sparks. Examples include:

- Welding

- Burning

- Flame cutting

- Brazing/soldering

- Plasma cutting

- Grinding

- Metal spraying

The Workplace Manager and/or Management OHS Nominee are to record identified hot work tasks in the ‘Hazard type’ column of OHS Risk Register.

## Hot work hazard identification

The Workplace Manager and/or Management OHS Nominee in consultation with employees and HSR are to identify and record hazards (e.g. fire and explosion) associated with the hot work tasks to be undertaken, in the ‘Hazard Description’ column of the OHS Risk Register.

## Record hot work risk controls

The Workplace Manager and/or Management OHS Nominee in consultation with employees and HSR are to identify, implement and record risk controls to manage hot work tasks in the OHS Risk Register, using the Hierarchy of controls. Examples include:

- Elimination – use pre-cast components e.g. purchase materials already joined.

- Substitution – conduct arc welding instead of flux-cored wire welding to reduce the risk of exposure to radiation and fumes

- Engineering – install local extraction ventilation systems, isolation of ignition sources, (see 3.3.1) and installation of welding bay screens

- Administration – develop and provide training in Safe Work Procedures for employees

- Personal Protective Equipment (PPE) – spark/fire resistant clothing or welding masks

For further information contact the OHS Advisory Service on 1300 074 715 or email safety@edumail.vic.gov.au.

## 3.3.1	Isolation of ignition sources

The Workplace Manager and/or Management OHS Nominee are to ensure that all hazardous areas/items surrounding the hot work area are isolated or otherwise controlled. This may include removing flammable materials and cordoning/screening off the area to prevent the ignition of any materials, contaminants, agents, or conditions that may be harmful to persons undertaking the tasks and others or property.

## 3.3.2	Fire watch personnel

The Workplace Manager, Management OHS Nominee, or Contractor is to ensure that a trained fire watch observer is appointed and present for the entire duration of the hot work task. The fire-watch observer must be able to operate fire and emergency equipment and wear PPE (e.g. welding mask).

## Employees undertaking hot work tasks

Employees undertaking hot work tasks, as part of their role, must complete a risk assessment using the Risk Assessment Template or equivalent template, see OHS Risk Management Procedure.

For support contact the OHS Advisory Service on 1300 074 715 or email: safety@edumail.vic.gov.au.

## Contractors undertaking hot work tasks

The Workplace Manager and/or Management OHS Nominee are to ensure that a contractor supplies a Safe Work Method Statement (SWMS) or equivalent template for any hot work related task, as mandated by the Department. The Workplace Manager and/or Management OHS Nominee must review and sign the SWMS or equivalent to verify it has been sighted and retain a copy. The SWMS at a minimum must include the following:

- removal of ignition sources and flammable material

- isolation of work area and signage

- emergency Procedures

- provision of PPE

- fire watch observer.

For further information see the Contractor OHS Management Procedure.

Defined terms

Terms defined within this Procedure can be located on the Department’s Defined Health and Safety Terms webpage. Defined roles will appear in bold.

Related references:

Occupational Health and Safety Act 2004

Occupational Health and Safety Regulations 2017

Related documentation:

Contractor OHS Management Procedure

OHS Induction and Training Procedure

OHS Training Planner/Register

OHS Risk Management Procedure

OHS Risk Register

Plant and Equipment Procedure

Risk Assessment Template

Safe Work Method Statement

Safe Work Procedure Template

Further assistance

Further information, advice or assistance on any matters related to hot works is available by contacting the OHS Advisory Service on ph. 1300 074 715 or email safety@edumail.vic.gov.au.